THERMODYNAMIC PROPERTIES OF WORKING FLUIDS

Thermodynamic Properties Obtained with CoolProp V6.6.0. Coolprop is a freely available library in Python [1]. This library includes functions for calculating the thermodynamic properties of 122 working fluids. The properties of the fluids: R245fa (Table 1), R600a (Table 2), 1234ze(Z) (Table 3) and 1234zf (Table 4) for analysis over the temperature and pressure range. The properties were estimated over a temperature and pressure range that allowed the results shown in the article to be recreated.

[1] Bell, I. H., Wronski, J., Quoilin, S., & Lemort, V. (2014). Pure and Pseudo-pure Fluid Thermophysical Property Evaluation and the Open-Source Thermophysical Property Library CoolProp. Industrial & Engineering Chemistry Research, 53(6), 2498–2508. doi:10.1021/ie4033999

Table 1. Thermodynamic Properties of fluid R245fa

| T [°C] | P [bar] | h [kJ/kg] | s [kJ/kg·K] |
| --- | --- | --- | --- |
| 40.0 | 2 | 436.65 | 1.7802 |
| 40.0 | 3 | 253.05 | 1.1808 |
| 40.0 | 4 | 253.08 | 1.1806 |
| 40.0 | 5 | 253.10 | 1.1805 |
| 40.0 | 6 | 253.12 | 1.1803 |
| 40.0 | 7 | 253.15 | 1.1801 |
| 40.0 | 8 | 253.17 | 1.1800 |
| 40.0 | 9 | 253.19 | 1.1798 |
| 40.0 | 10 | 253.22 | 1.1796 |
| 45.4 | 2 | 441.69 | 1.7962 |
| 45.4 | 3 | 260.36 | 1.2039 |
| 45.4 | 4 | 260.38 | 1.2038 |
| 45.4 | 5 | 260.40 | 1.2036 |
| 45.4 | 6 | 260.42 | 1.2034 |
| 45.4 | 7 | 260.44 | 1.2032 |
| 45.4 | 8 | 260.46 | 1.2030 |
| 45.4 | 9 | 260.48 | 1.2029 |
| 45.4 | 10 | 260.51 | 1.2027 |
| 50.7 | 2 | 446.76 | 1.8120 |
| 50.7 | 3 | 444.47 | 1.7815 |
| 50.7 | 4 | 267.76 | 1.2268 |
| 50.7 | 5 | 267.78 | 1.2266 |
| 50.7 | 6 | 267.80 | 1.2264 |
| 50.7 | 7 | 267.82 | 1.2262 |
| 50.7 | 8 | 267.84 | 1.2260 |
| 50.7 | 9 | 267.85 | 1.2258 |
| 50.7 | 10 | 267.87 | 1.2256 |
| 56.1 | 2 | 451.87 | 1.8276 |
| 56.1 | 3 | 449.71 | 1.7975 |
| 56.1 | 4 | 447.39 | 1.7743 |
| 56.1 | 5 | 275.25 | 1.2494 |
| 56.1 | 6 | 275.26 | 1.2492 |
| 56.1 | 7 | 275.28 | 1.2490 |
| 56.1 | 8 | 275.29 | 1.2488 |
| 56.1 | 9 | 275.31 | 1.2486 |
| 56.1 | 10 | 275.32 | 1.2484 |
| 61.5 | 2 | 457.01 | 1.8431 |
| 61.5 | 3 | 454.98 | 1.8134 |
| 61.5 | 4 | 452.81 | 1.7906 |
| 61.5 | 5 | 282.81 | 1.2722 |
| 61.5 | 6 | 282.82 | 1.2720 |
| 61.5 | 7 | 282.83 | 1.2718 |
| 61.5 | 8 | 282.84 | 1.2716 |
| 61.5 | 9 | 282.85 | 1.2714 |
| 61.5 | 10 | 282.86 | 1.2712 |
| 66.8 | 2 | 462.19 | 1.8585 |
| 66.8 | 3 | 460.27 | 1.8291 |
| 66.8 | 4 | 458.23 | 1.8067 |
| 66.8 | 5 | 456.04 | 1.7878 |
| 66.8 | 6 | 290.48 | 1.2947 |
| 66.8 | 7 | 290.48 | 1.2945 |
| 66.8 | 8 | 290.49 | 1.2942 |
| 66.8 | 9 | 290.49 | 1.2940 |
| 66.8 | 10 | 290.50 | 1.2938 |
| 72.2 | 2 | 467.41 | 1.8737 |
| 72.2 | 3 | 465.59 | 1.8446 |
| 72.2 | 4 | 463.67 | 1.8225 |
| 72.2 | 5 | 461.62 | 1.8041 |
| 72.2 | 6 | 459.42 | 1.7879 |
| 72.2 | 7 | 298.24 | 1.3171 |
| 72.2 | 8 | 298.24 | 1.3169 |
| 72.2 | 9 | 298.24 | 1.3166 |
| 72.2 | 10 | 298.24 | 1.3164 |
| 77.5 | 2 | 472.66 | 1.8888 |
| 77.5 | 3 | 470.94 | 1.8600 |
| 77.5 | 4 | 469.13 | 1.8382 |
| 77.5 | 5 | 467.20 | 1.8202 |
| 77.5 | 6 | 465.16 | 1.8043 |
| 77.5 | 7 | 462.95 | 1.7899 |
| 77.5 | 8 | 306.10 | 1.3395 |
| 77.5 | 9 | 306.09 | 1.3392 |
| 77.5 | 10 | 306.09 | 1.3389 |
| 82.9 | 2 | 477.95 | 1.9038 |
| 82.9 | 3 | 476.32 | 1.8752 |
| 82.9 | 4 | 474.60 | 1.8537 |
| 82.9 | 5 | 472.80 | 1.8360 |
| 82.9 | 6 | 470.88 | 1.8205 |
| 82.9 | 7 | 468.83 | 1.8065 |
| 82.9 | 8 | 466.64 | 1.7934 |
| 82.9 | 9 | 314.07 | 1.3618 |
| 82.9 | 10 | 314.06 | 1.3615 |
| 88.3 | 2 | 483.28 | 1.9186 |
| 88.3 | 3 | 481.73 | 1.8903 |
| 88.3 | 4 | 480.11 | 1.8690 |
| 88.3 | 5 | 478.40 | 1.8516 |
| 88.3 | 6 | 476.60 | 1.8365 |
| 88.3 | 7 | 474.69 | 1.8228 |
| 88.3 | 8 | 472.66 | 1.8101 |
| 88.3 | 9 | 470.47 | 1.7980 |
| 88.3 | 10 | 322.17 | 1.3841 |
| 93.6 | 2 | 488.65 | 1.9334 |
| 93.6 | 3 | 487.18 | 1.9052 |
| 93.6 | 4 | 485.64 | 1.8842 |
| 93.6 | 5 | 484.02 | 1.8671 |
| 93.6 | 6 | 482.33 | 1.8522 |
| 93.6 | 7 | 480.54 | 1.8389 |
| 93.6 | 8 | 478.65 | 1.8266 |
| 93.6 | 9 | 476.63 | 1.8150 |
| 93.6 | 10 | 474.46 | 1.8037 |
| 99.0 | 2 | 494.07 | 1.9480 |
| 99.0 | 3 | 492.66 | 1.9201 |
| 99.0 | 4 | 491.19 | 1.8993 |
| 99.0 | 5 | 489.66 | 1.8823 |
| 99.0 | 6 | 488.06 | 1.8677 |
| 99.0 | 7 | 486.39 | 1.8547 |
| 99.0 | 8 | 484.62 | 1.8428 |
| 99.0 | 9 | 482.75 | 1.8315 |
| 99.0 | 10 | 480.75 | 1.8207 |

Table 2. Thermodynamic Properties of fluid R600a

| T [°C] | P [bar] | h [kJ/kg] | s [kJ/kg·K] |
| --- | --- | --- | --- |
| 40.0 | 5 | 609.14 | 2.3329 |
| 40.0 | 6 | 296.30 | 1.3259 |
| 40.0 | 7 | 296.34 | 1.3254 |
| 40.0 | 8 | 296.38 | 1.3250 |
| 40.0 | 9 | 296.41 | 1.3245 |
| 40.0 | 10 | 296.45 | 1.3240 |
| 40.0 | 11 | 296.49 | 1.3235 |
| 40.0 | 12 | 296.53 | 1.3230 |
| 40.0 | 13 | 296.56 | 1.3226 |
| 40.0 | 14 | 296.60 | 1.3221 |
| 40.0 | 15 | 296.64 | 1.3216 |
| 40.0 | 16 | 296.68 | 1.3211 |
| 45.4 | 5 | 619.38 | 2.3653 |
| 45.4 | 6 | 615.25 | 2.3296 |
| 45.4 | 7 | 310.03 | 1.3688 |
| 45.4 | 8 | 310.06 | 1.3683 |
| 45.4 | 9 | 310.09 | 1.3678 |
| 45.4 | 10 | 310.11 | 1.3673 |
| 45.4 | 11 | 310.14 | 1.3667 |
| 45.4 | 12 | 310.17 | 1.3662 |
| 45.4 | 13 | 310.20 | 1.3657 |
| 45.4 | 14 | 310.23 | 1.3652 |
| 45.4 | 15 | 310.26 | 1.3647 |
| 45.4 | 16 | 310.29 | 1.3642 |
| 50.7 | 5 | 629.68 | 2.3974 |
| 50.7 | 6 | 625.81 | 2.3625 |
| 50.7 | 7 | 323.97 | 1.4122 |
| 50.7 | 8 | 323.98 | 1.4116 |
| 50.7 | 9 | 323.99 | 1.4111 |
| 50.7 | 10 | 324.01 | 1.4105 |
| 50.7 | 11 | 324.03 | 1.4100 |
| 50.7 | 12 | 324.04 | 1.4094 |
| 50.7 | 13 | 324.06 | 1.4089 |
| 50.7 | 14 | 324.08 | 1.4084 |
| 50.7 | 15 | 324.10 | 1.4078 |
| 50.7 | 16 | 324.12 | 1.4073 |
| 56.1 | 5 | 640.04 | 2.4291 |
| 56.1 | 6 | 636.40 | 2.3949 |
| 56.1 | 7 | 632.51 | 2.3641 |
| 56.1 | 8 | 338.16 | 1.4550 |
| 56.1 | 9 | 338.16 | 1.4544 |
| 56.1 | 10 | 338.16 | 1.4538 |
| 56.1 | 11 | 338.16 | 1.4533 |
| 56.1 | 12 | 338.17 | 1.4527 |
| 56.1 | 13 | 338.17 | 1.4521 |
| 56.1 | 14 | 338.18 | 1.4515 |
| 56.1 | 15 | 338.18 | 1.4509 |
| 56.1 | 16 | 338.19 | 1.4504 |
| 61.5 | 5 | 650.49 | 2.4606 |
| 61.5 | 6 | 647.04 | 2.4270 |
| 61.5 | 7 | 643.39 | 2.3969 |
| 61.5 | 8 | 639.49 | 2.3690 |
| 61.5 | 9 | 352.60 | 1.4979 |
| 61.5 | 10 | 352.58 | 1.4973 |
| 61.5 | 11 | 352.57 | 1.4967 |
| 61.5 | 12 | 352.56 | 1.4960 |
| 61.5 | 13 | 352.55 | 1.4954 |
| 61.5 | 14 | 352.54 | 1.4948 |
| 61.5 | 15 | 352.53 | 1.4942 |
| 61.5 | 16 | 352.52 | 1.4935 |
| 66.8 | 5 | 661.02 | 2.4918 |
| 66.8 | 6 | 657.75 | 2.4587 |
| 66.8 | 7 | 654.31 | 2.4292 |
| 66.8 | 8 | 650.66 | 2.4021 |
| 66.8 | 9 | 646.76 | 2.3766 |
| 66.8 | 10 | 642.56 | 2.3521 |
| 66.8 | 11 | 367.28 | 1.5403 |
| 66.8 | 12 | 367.25 | 1.5396 |
| 66.8 | 13 | 367.22 | 1.5389 |
| 66.8 | 14 | 367.19 | 1.5382 |
| 66.8 | 15 | 367.16 | 1.5375 |
| 66.8 | 16 | 367.14 | 1.5369 |
| 72.2 | 5 | 671.63 | 2.5228 |
| 72.2 | 6 | 668.53 | 2.4902 |
| 72.2 | 7 | 665.28 | 2.4613 |
| 72.2 | 8 | 661.84 | 2.4348 |
| 72.2 | 9 | 658.20 | 2.4100 |
| 72.2 | 10 | 654.31 | 2.3864 |
| 72.2 | 11 | 650.13 | 2.3634 |
| 72.2 | 12 | 382.27 | 1.5834 |
| 72.2 | 13 | 382.22 | 1.5827 |
| 72.2 | 14 | 382.17 | 1.5819 |
| 72.2 | 15 | 382.12 | 1.5812 |
| 72.2 | 16 | 382.07 | 1.5804 |
| 77.5 | 5 | 682.35 | 2.5536 |
| 77.5 | 6 | 679.39 | 2.5214 |
| 77.5 | 7 | 676.30 | 2.4929 |
| 77.5 | 8 | 673.06 | 2.4670 |
| 77.5 | 9 | 669.65 | 2.4429 |
| 77.5 | 10 | 666.02 | 2.4201 |
| 77.5 | 11 | 662.16 | 2.3980 |
| 77.5 | 12 | 658.01 | 2.3764 |
| 77.5 | 13 | 397.60 | 1.6269 |
| 77.5 | 14 | 397.51 | 1.6260 |
| 77.5 | 15 | 397.43 | 1.6252 |
| 77.5 | 16 | 397.36 | 1.6244 |
| 82.9 | 5 | 693.16 | 2.5842 |
| 82.9 | 6 | 690.34 | 2.5524 |
| 82.9 | 7 | 687.40 | 2.5244 |
| 82.9 | 8 | 684.33 | 2.4989 |
| 82.9 | 9 | 681.11 | 2.4754 |
| 82.9 | 10 | 677.72 | 2.4532 |
| 82.9 | 11 | 674.13 | 2.4319 |
| 82.9 | 12 | 670.31 | 2.4112 |
| 82.9 | 13 | 666.20 | 2.3907 |
| 82.9 | 14 | 661.73 | 2.3702 |
| 82.9 | 15 | 413.18 | 1.6698 |
| 82.9 | 16 | 413.06 | 1.6688 |
| 88.3 | 5 | 704.08 | 2.6146 |
| 88.3 | 6 | 701.38 | 2.5832 |
| 88.3 | 7 | 698.58 | 2.5555 |
| 88.3 | 8 | 695.67 | 2.5305 |
| 88.3 | 9 | 692.62 | 2.5075 |
| 88.3 | 10 | 689.43 | 2.4858 |
| 88.3 | 11 | 686.07 | 2.4652 |
| 88.3 | 12 | 682.52 | 2.4452 |
| 88.3 | 13 | 678.75 | 2.4257 |
| 88.3 | 14 | 674.70 | 2.4064 |
| 88.3 | 15 | 670.31 | 2.3868 |
| 88.3 | 16 | 429.27 | 1.7140 |
| 93.6 | 5 | 715.10 | 2.6449 |
| 93.6 | 6 | 712.52 | 2.6138 |
| 93.6 | 7 | 709.84 | 2.5864 |
| 93.6 | 8 | 707.07 | 2.5618 |
| 93.6 | 9 | 704.18 | 2.5392 |
| 93.6 | 10 | 701.17 | 2.5180 |
| 93.6 | 11 | 698.02 | 2.4980 |
| 93.6 | 12 | 694.70 | 2.4787 |
| 93.6 | 13 | 691.20 | 2.4599 |
| 93.6 | 14 | 687.49 | 2.4415 |
| 93.6 | 15 | 683.51 | 2.4231 |
| 93.6 | 16 | 679.22 | 2.4046 |
| 99.0 | 5 | 726.24 | 2.6750 |
| 99.0 | 6 | 723.76 | 2.6442 |
| 99.0 | 7 | 721.20 | 2.6172 |
| 99.0 | 8 | 718.55 | 2.5929 |
| 99.0 | 9 | 715.80 | 2.5707 |
| 99.0 | 10 | 712.95 | 2.5499 |
| 99.0 | 11 | 709.98 | 2.5303 |
| 99.0 | 12 | 706.87 | 2.5116 |
| 99.0 | 13 | 703.61 | 2.4935 |
| 99.0 | 14 | 700.17 | 2.4758 |
| 99.0 | 15 | 696.53 | 2.4583 |
| 99.0 | 16 | 692.64 | 2.4409 |

Table 3. Thermodynamic Properties of fluid 1234ze(Z)

| T [°C] | P [bar] | h [kJ/kg] | s [kJ/kg·K] |
| --- | --- | --- | --- |
| 40.0 | 2 | 450.05 | 1.8338 |
| 40.0 | 3 | 250.53 | 1.1718 |
| 40.0 | 4 | 250.55 | 1.1716 |
| 40.0 | 5 | 250.57 | 1.1714 |
| 40.0 | 6 | 250.59 | 1.1712 |
| 40.0 | 7 | 250.61 | 1.1710 |
| 40.0 | 8 | 250.63 | 1.1708 |
| 40.0 | 9 | 250.65 | 1.1706 |
| 40.0 | 10 | 250.67 | 1.1704 |
| 40.0 | 11 | 250.69 | 1.1702 |
| 45.4 | 2 | 454.96 | 1.8494 |
| 45.4 | 3 | 451.91 | 1.8125 |
| 45.4 | 4 | 257.67 | 1.1942 |
| 45.4 | 5 | 257.69 | 1.1940 |
| 45.4 | 6 | 257.71 | 1.1937 |
| 45.4 | 7 | 257.73 | 1.1935 |
| 45.4 | 8 | 257.74 | 1.1933 |
| 45.4 | 9 | 257.76 | 1.1931 |
| 45.4 | 10 | 257.78 | 1.1929 |
| 45.4 | 11 | 257.80 | 1.1927 |
| 50.7 | 2 | 459.91 | 1.8648 |
| 50.7 | 3 | 457.04 | 1.8285 |
| 50.7 | 4 | 264.90 | 1.2167 |
| 50.7 | 5 | 264.91 | 1.2164 |
| 50.7 | 6 | 264.93 | 1.2162 |
| 50.7 | 7 | 264.94 | 1.2160 |
| 50.7 | 8 | 264.95 | 1.2158 |
| 50.7 | 9 | 264.97 | 1.2155 |
| 50.7 | 10 | 264.98 | 1.2153 |
| 50.7 | 11 | 264.99 | 1.2151 |
| 56.1 | 2 | 464.88 | 1.8800 |
| 56.1 | 3 | 462.19 | 1.8443 |
| 56.1 | 4 | 459.31 | 1.8167 |
| 56.1 | 5 | 272.24 | 1.2389 |
| 56.1 | 6 | 272.25 | 1.2386 |
| 56.1 | 7 | 272.25 | 1.2384 |
| 56.1 | 8 | 272.26 | 1.2381 |
| 56.1 | 9 | 272.27 | 1.2379 |
| 56.1 | 10 | 272.28 | 1.2377 |
| 56.1 | 11 | 272.29 | 1.2374 |
| 61.5 | 2 | 469.89 | 1.8951 |
| 61.5 | 3 | 467.35 | 1.8598 |
| 61.5 | 4 | 464.65 | 1.8328 |
| 61.5 | 5 | 461.76 | 1.8099 |
| 61.5 | 6 | 279.68 | 1.2610 |
| 61.5 | 7 | 279.68 | 1.2608 |
| 61.5 | 8 | 279.68 | 1.2605 |
| 61.5 | 9 | 279.69 | 1.2602 |
| 61.5 | 10 | 279.69 | 1.2600 |
| 61.5 | 11 | 279.70 | 1.2597 |
| 66.8 | 2 | 474.93 | 1.9100 |
| 66.8 | 3 | 472.52 | 1.8752 |
| 66.8 | 4 | 469.99 | 1.8486 |
| 66.8 | 5 | 467.30 | 1.8263 |
| 66.8 | 6 | 464.41 | 1.8064 |
| 66.8 | 7 | 287.22 | 1.2831 |
| 66.8 | 8 | 287.22 | 1.2828 |
| 66.8 | 9 | 287.22 | 1.2826 |
| 66.8 | 10 | 287.21 | 1.2823 |
| 66.8 | 11 | 287.21 | 1.2820 |
| 72.2 | 2 | 480.00 | 1.9248 |
| 72.2 | 3 | 477.73 | 1.8904 |
| 72.2 | 4 | 475.34 | 1.8642 |
| 72.2 | 5 | 472.81 | 1.8424 |
| 72.2 | 6 | 470.13 | 1.8231 |
| 72.2 | 7 | 467.26 | 1.8054 |
| 72.2 | 8 | 294.88 | 1.3052 |
| 72.2 | 9 | 294.87 | 1.3049 |
| 72.2 | 10 | 294.86 | 1.3046 |
| 72.2 | 11 | 294.85 | 1.3043 |
| 77.5 | 2 | 485.11 | 1.9395 |
| 77.5 | 3 | 482.95 | 1.9054 |
| 77.5 | 4 | 480.70 | 1.8796 |
| 77.5 | 5 | 478.32 | 1.8582 |
| 77.5 | 6 | 475.82 | 1.8395 |
| 77.5 | 7 | 473.15 | 1.8224 |
| 77.5 | 8 | 470.30 | 1.8063 |
| 77.5 | 9 | 302.66 | 1.3273 |
| 77.5 | 10 | 302.64 | 1.3270 |
| 77.5 | 11 | 302.63 | 1.3267 |
| 82.9 | 2 | 490.26 | 1.9541 |
| 82.9 | 3 | 488.21 | 1.9203 |
| 82.9 | 4 | 486.07 | 1.8948 |
| 82.9 | 5 | 483.83 | 1.8738 |
| 82.9 | 6 | 481.48 | 1.8555 |
| 82.9 | 7 | 479.00 | 1.8389 |
| 82.9 | 8 | 476.36 | 1.8234 |
| 82.9 | 9 | 473.54 | 1.8086 |
| 82.9 | 10 | 310.58 | 1.3494 |
| 82.9 | 11 | 310.55 | 1.3491 |
| 88.3 | 2 | 495.45 | 1.9685 |
| 88.3 | 3 | 493.50 | 1.9350 |
| 88.3 | 4 | 491.46 | 1.9098 |
| 88.3 | 5 | 489.35 | 1.8892 |
| 88.3 | 6 | 487.14 | 1.8713 |
| 88.3 | 7 | 484.82 | 1.8551 |
| 88.3 | 8 | 482.36 | 1.8402 |
| 88.3 | 9 | 479.76 | 1.8259 |
| 88.3 | 10 | 476.97 | 1.8122 |
| 88.3 | 11 | 318.64 | 1.3716 |
| 93.6 | 2 | 500.68 | 1.9829 |
| 93.6 | 3 | 498.81 | 1.9496 |
| 93.6 | 4 | 496.88 | 1.9247 |
| 93.6 | 5 | 494.87 | 1.9044 |
| 93.6 | 6 | 492.79 | 1.8868 |
| 93.6 | 7 | 490.61 | 1.8710 |
| 93.6 | 8 | 488.32 | 1.8565 |
| 93.6 | 9 | 485.90 | 1.8428 |
| 93.6 | 10 | 483.34 | 1.8297 |
| 93.6 | 11 | 480.59 | 1.8168 |
| 99.0 | 2 | 505.94 | 1.9972 |
| 99.0 | 3 | 504.16 | 1.9641 |
| 99.0 | 4 | 502.32 | 1.9394 |
| 99.0 | 5 | 500.41 | 1.9194 |
| 99.0 | 6 | 498.44 | 1.9021 |
| 99.0 | 7 | 496.38 | 1.8867 |
| 99.0 | 8 | 494.23 | 1.8725 |
| 99.0 | 9 | 491.98 | 1.8593 |
| 99.0 | 10 | 489.61 | 1.8466 |
| 99.0 | 11 | 487.10 | 1.8344 |

Table 4. Thermodynamic Properties of fluid 1234zf

| T [°C] | P [bar] | h [kJ/kg] | s [kJ/kg·K] |
| --- | --- | --- | --- |
| 40.0 | 2 | 441.34 | 1.8979 |
| 40.0 | 3 | 439.47 | 1.8585 |
| 40.0 | 4 | 437.49 | 1.8290 |
| 40.0 | 5 | 435.41 | 1.8048 |
| 40.0 | 6 | 433.21 | 1.7837 |
| 40.0 | 7 | 430.85 | 1.7646 |
| 40.0 | 8 | 428.31 | 1.7468 |
| 40.0 | 9 | 258.80 | 1.1984 |
| 45.4 | 2 | 446.72 | 1.9150 |
| 45.4 | 3 | 444.95 | 1.8759 |
| 45.4 | 4 | 443.09 | 1.8467 |
| 45.4 | 5 | 441.13 | 1.8229 |
| 45.4 | 6 | 439.07 | 1.8023 |
| 45.4 | 7 | 436.89 | 1.7838 |
| 45.4 | 8 | 434.55 | 1.7666 |
| 45.4 | 9 | 432.05 | 1.7503 |
| 50.7 | 2 | 452.16 | 1.9319 |
| 50.7 | 3 | 450.47 | 1.8930 |
| 50.7 | 4 | 448.71 | 1.8642 |
| 50.7 | 5 | 446.87 | 1.8407 |
| 50.7 | 6 | 444.94 | 1.8205 |
| 50.7 | 7 | 442.90 | 1.8025 |
| 50.7 | 8 | 440.75 | 1.7859 |
| 50.7 | 9 | 438.45 | 1.7703 |
| 56.1 | 2 | 457.64 | 1.9487 |
| 56.1 | 3 | 456.04 | 1.9101 |
| 56.1 | 4 | 454.37 | 1.8815 |
| 56.1 | 5 | 452.63 | 1.8584 |
| 56.1 | 6 | 450.81 | 1.8385 |
| 56.1 | 7 | 448.91 | 1.8209 |
| 56.1 | 8 | 446.91 | 1.8048 |
| 56.1 | 9 | 444.79 | 1.7897 |
| 61.5 | 2 | 463.18 | 1.9654 |
| 61.5 | 3 | 461.65 | 1.9270 |
| 61.5 | 4 | 460.07 | 1.8987 |
| 61.5 | 5 | 458.42 | 1.8758 |
| 61.5 | 6 | 456.71 | 1.8563 |
| 61.5 | 7 | 454.92 | 1.8390 |
| 61.5 | 8 | 453.05 | 1.8233 |
| 61.5 | 9 | 451.08 | 1.8086 |
| 66.8 | 2 | 468.77 | 1.9819 |
| 66.8 | 3 | 467.31 | 1.9438 |
| 66.8 | 4 | 465.80 | 1.9157 |
| 66.8 | 5 | 464.24 | 1.8931 |
| 66.8 | 6 | 462.62 | 1.8738 |
| 66.8 | 7 | 460.93 | 1.8568 |
| 66.8 | 8 | 459.18 | 1.8414 |
| 66.8 | 9 | 457.34 | 1.8272 |
| 72.2 | 2 | 474.42 | 1.9984 |
| 72.2 | 3 | 473.02 | 1.9604 |
| 72.2 | 4 | 471.58 | 1.9326 |
| 72.2 | 5 | 470.09 | 1.9101 |
| 72.2 | 6 | 468.55 | 1.8911 |
| 72.2 | 7 | 466.96 | 1.8744 |
| 72.2 | 8 | 465.31 | 1.8593 |
| 72.2 | 9 | 463.59 | 1.8454 |
| 77.5 | 2 | 480.11 | 2.0148 |
| 77.5 | 3 | 478.77 | 1.9770 |
| 77.5 | 4 | 477.40 | 1.9493 |
| 77.5 | 5 | 475.98 | 1.9271 |
| 77.5 | 6 | 474.51 | 1.9083 |
| 77.5 | 7 | 473.01 | 1.8918 |
| 77.5 | 8 | 471.44 | 1.8770 |
| 77.5 | 9 | 469.82 | 1.8633 |
| 82.9 | 2 | 485.86 | 2.0311 |
| 82.9 | 3 | 484.58 | 1.9934 |
| 82.9 | 4 | 483.26 | 1.9659 |
| 82.9 | 5 | 481.90 | 1.9438 |
| 82.9 | 6 | 480.51 | 1.9252 |
| 82.9 | 7 | 479.07 | 1.9090 |
| 82.9 | 8 | 477.59 | 1.8944 |
| 82.9 | 9 | 476.06 | 1.8810 |
| 88.3 | 2 | 491.67 | 2.0472 |
| 88.3 | 3 | 490.43 | 2.0097 |
| 88.3 | 4 | 489.16 | 1.9823 |
| 88.3 | 5 | 487.86 | 1.9604 |
| 88.3 | 6 | 486.53 | 1.9420 |
| 88.3 | 7 | 485.16 | 1.9259 |
| 88.3 | 8 | 483.76 | 1.9115 |
| 88.3 | 9 | 482.31 | 1.8984 |
| 93.6 | 2 | 497.52 | 2.0633 |
| 93.6 | 3 | 496.33 | 2.0259 |
| 93.6 | 4 | 495.11 | 1.9987 |
| 93.6 | 5 | 493.87 | 1.9769 |
| 93.6 | 6 | 492.59 | 1.9587 |
| 93.6 | 7 | 491.29 | 1.9427 |
| 93.6 | 8 | 489.95 | 1.9285 |
| 93.6 | 9 | 488.57 | 1.9156 |
| 99.0 | 2 | 503.43 | 2.0793 |
| 99.0 | 3 | 502.28 | 2.0420 |
| 99.0 | 4 | 501.11 | 2.0149 |
| 99.0 | 5 | 499.91 | 1.9933 |
| 99.0 | 6 | 498.69 | 1.9752 |
| 99.0 | 7 | 497.44 | 1.9594 |
| 99.0 | 8 | 496.16 | 1.9454 |
| 99.0 | 9 | 494.85 | 1.9326 |